# Project 2 – MANE 6710 Numerical Design Optimization

Unique ID: 5284

Table of Contents

Introduction	2

Choice of Design Variable	2

Objective Function and Constraints	2

Calculation of Volume	3

Calculation of Constraints	3

Setting up Linear Inequality constraints	3

Setting up Nonlinear constraints	3

Setting up Optimization Problem	4

Results	6

Appendix	8

List of Figures

Figure 1. Cross- sectional Profile of the spar	2

Figure 2. Initial Design Parameterization	5

Figure 3. Optimal design of Spar Cross-section	5

Figure 4. First Order Optimality condition	6

Figure 5. Mass estimates (v) number of elements	6

Figure 6. Optimized Spar Performance	7

# Introduction

The main objective of this project is to design the cross-sectional profile of a spar of an aircraft wing. The idea is to arrive at a design which leads to the least weight possible for the structure given certain design parameters. The optimal design that comes as a result should not fail under the loading conditions and in fact it must be manufacturable.

# Choice of Design Variable

With the cross section fixed to be a circular annulus, the most obvious choice of design variables are the Radii of the annulus along the length of the spar. The parameterization of design variable chosen in this project is as follows: R=Rout1 Rin1 Rout2 Rin2…. Routn Rinn'-Routi, Rini i:1(1)n

The length of the spar can be discretized into n elements (Nelem) and with a pair of design variables {Routi, Rini} at each node as shown in Figure 1.

Figure 1. Cross- sectional Profile of the spar

# Objective Function and Constraints

The objective of this project is to optimize the design of the spar’s cross-section that has the least weight possible.

This cross-sectional profile needs to adhere to certain design parameters such as:

- Semi Span Length – 7.5 m

- Shape of Cross section – Circular Annulus

- Material – Carbon Fiber composite – {Density ρ=1600 kgm3 ;Young’s modulus E=70 GPa; Yield Compressive/Tensile stress σmax=600 MPa}

- Aircraft operational weight – 500 kg

- Loading – 2.5 g with force distribution linearly varying along the length (maximum load at the root and no load at the tip)

With these parameters, the objective function will be the Mass or the Weight of the spar which can be calculated as m=ρV kg;V-volume m3, ρ-density (kgm3). Hence this design problem can now be formulated as:

| minRρV(R) | Equation 1 |
| --- | --- |

This minimization problem is subject to constraints that stem from the manufacturing side of the spar, and they are:

- c1R= Rout≤5cm along the entire length of the spar

- c2R= Rin≥1cm along the entire length of the spar

- c3R= Rout-Rin≥2.5mm along the entire length of the spar

- c4R= σ≤σmax along the entire length of the spar

MATLAB’s in-built optimizer fmincon(obj,a0,A,b,Aeq,beq,lb,ub,Nonlinear,options) is used to perform the optimization of spar cross-section.

## Calculation of Volume

Volume is generally calculated by the integral 0LA(x) dx where, A(x) is the cross-section along the length of the spar. With very fine discretization of the length of spar into Nelem, this integral can be approximated as:

| i=1Nelem12πRouti2-Rini2+πRouti+12-Rini+12δxi | Equation 2 |
| --- | --- |

This calculates the average area of the cross-section on two adjacent node points and multiplies it with the distance between these two node points δxi. This is a variational form of trapezoidal integration technique.

## Calculation of Constraints

### Setting up Linear Inequality constraints

Constraints c1R, c2R, c3(R) are set up as Linear Inequality constraints of the format fmincon requires it in - AR≤b where A is the matrix that contains the linear mapping between the design variables R and constraint values b.

### Setting up Nonlinear constraints

This is the most important aspect of this project as the computation of normal stress at each cross-section requires the use of numerical methods like Finite Element Methods to calculate it. The Area moment of Inertia for each cross-section along the length is,

| Iyyi= 14π(Routi4-Rini4) | Equation 3 |
| --- | --- |

Then, the force which varies linearly along the length of the wing is calculated at each Node point as

| 0Lf dx= 2.5*Operational Load(N)2 ;Fi=fxi N | Equation 3 |
| --- | --- |

The operational load considered in this problem is Operational Weight - 500kg ×9.81 m/s2

Using the Area Moment of Inertia and the force on each node, the vertical displacement and the angular displacement of the wing at each node is calculated using the FEM (CalcBeamDisplacement.m). Then, it gets easier to solve for the Normal stress σi along the length of the spar. This is calculated using CalcBeamStress.m function. Most important aspect of any optimization problem is to perform optimization in scales that are the same for all the parameters that are involved in this process. Since, the calculated maximum σi at each node is at large scales compared to the value of the objective function or the design variables, this has been normalized and the final nonlinear inequality constraint c4R is written as

| ¦σi¦σmax≤1 | Equation 4 |
| --- | --- |

# Setting up Optimization Problem

With the objective and the constraints set up, it is now important to calculate accurate gradients of the objective and the constraints for successful optimization of the design. Complex step gradient method is used to compute the gradients of the objective and the inequality constraints. With a very small complex step h=10-60 ,the round off errors caused by the central difference approximation can be avoided.

Two functions, Calc_objGradX and Calc_consJac(X,Nnodes,L,E,F,σmax) are used to compute the gradients of the objective function and the nonlinear constraint function.

Number of elements - 100

Initial point chosen - 0.030.015     0.030.0150.03 ⋯⋯⋯0.030.015202×1'

Options chosen for fmincon –

- Choice of Algorithm – ‘Sequential Quadratic Programming (SQP)’

- Gradients – Provided and computed using Complex Step Algorithm

Initial weight of the spar was 25.446900494077372 kg

Initial Design choice is shown in Figure 2

Figure 2. Initial Design Parameterization

After optimization is performed, the optimal profile of the cross-section is shown in Figure 3

Figure 3. Optimal design of Spar Cross-section

Figure 4. First Order Optimality condition

Figure 4 shows the first order optimality condition to decrease and this conveys the fact that the optimizer has converged to a local minimizer which also satisfies the constraints.

The final weight of this optimal design is 5.273609498808303 kg. This is a 79.28% reduction of weight from the initial design.

# Results

A convergence analysis was performed to find if the mass estimate computed in this project improves as the number of elements (Nelem) are increased. As more cross-sections are added along the length elements, approximate volume calculated using the trapezoidal integral method improves as δxi decreases and a small increase in the mass computed can be noticed as shown in Figure 5.

Figure 5. Mass estimates (v) number of elements

Optimized profile undergoes the following deformation, stress under the application of a linearly varying force along the length of the semi spar of the wing as shown in Figure 6.

Figure 6. Optimized Spar Performance

This profile for the spar not only has decreased its mass but has also adhered to all the manufacturing constraints specified for the design of the spar. This is the final optimized profile achieved using this parameterization for the choice of design variables Routi, Rini i:11n.

# Appendix

The MATLAB code written for this project has been attached in this section.