# Project 2 – MANE 6710 Numerical Design Optimization

Unique ID: 5284

Table of Contents

Introduction	2

Choice of Design Variable	2

Objective Function and Constraints	2

Calculation of Volume	3

Calculation of Constraints	3

Setting up Linear Inequality constraints	3

Setting up Nonlinear constraints	3

Loading with Uncertainty	4

Setting up Optimization Problem	6

Results	8

Appendix	10

List of Figures

Figure 1. Cross- sectional Profile of the spar	2

Figure 2. Initial Design Parameterization	6

Figure 3. Optimal design of Spar Cross-section	7

Figure 4. First Order Optimality condition	7

Figure 5. Mass estimates (v) number of elements	8

Figure 6. Displacement under uncertain loading	8

Figure 7. Stress under uncertain loading	8

# Introduction

The main objective of this project is to design the cross-sectional profile of a spar of an aircraft wing. The idea is to arrive at a design which leads to the least weight possible for the structure given certain design parameters. The optimal design that comes as a result should not fail under the loading conditions and in fact it must be manufacturable. Additionally, the loading conditions are not deterministic, and it is perturbed by uncertainties.

# Choice of Design Variable

With the cross section fixed to be a circular annulus, the most obvious choice of design variables are the Radii of the annulus along the length of the spar. The parameterization of design variable chosen in this project is as follows: R=Rout1 Rin1 Rout2 Rin2…. Routn Rinn'-Routi, Rini i:1(1)n

The length of the spar can be discretized into n elements (Nelem) and with a pair of design variables {Routi, Rini} at each node as shown in Figure 1.

Figure 1. Cross- sectional Profile of the spar

# Objective Function and Constraints

The objective of this project is to optimize the design of the spar’s cross-section that has the least weight possible.

This cross-sectional profile needs to adhere to certain design parameters such as:

- Semi Span Length – 7.5 m

- Shape of Cross section – Circular Annulus

- Material – Carbon Fiber composite – {Density ρ=1600 kgm3 ;Young’s modulus E=70 GPa; Yield Compressive/Tensile stress σmax=600 MPa}

- Aircraft operational weight – 500 kg

- Nominal Loading – 2.5 g with force distribution linearly varying along the length (maximum load at the root and no load at the tip)

- Loading with uncertainty – Uncertain loading is modelled and described in section “Loading with uncertainty”.

With these parameters, the objective function will be the Mass or the Weight of the spar which can be calculated as m=ρV kg;V-volume m3, ρ-density (kgm3). Hence this design problem can now be formulated as:

| minRρV(R) | Equation 1 |
| --- | --- |

This minimization problem is subject to constraints that stem from the manufacturing side of the spar, and they are:

- c1R= Rout≤5cm along the entire length of the spar

- c2R= Rin≥1cm along the entire length of the spar

- c3R= Rout-Rin≥2.5mm along the entire length of the spar

- c4R= Eσ+6σ2- Eσ2≤σmax(yield stress) – this condition arises

MATLAB’s in-built optimizer fmincon(obj,a0,A,b,Aeq,beq,lb,ub,Nonlinear,options) is used to perform the optimization of spar cross-section.

## Calculation of Volume

Volume is generally calculated by the integral 0LA(x) dx where, A(x) is the cross-section along the length of the spar. With very fine discretization of the length of spar into Nelem, this integral can be approximated as:

| i=1Nelem12πRouti2-Rini2+πRouti+12-Rini+12δxi | Equation 2 |
| --- | --- |

This calculates the average area of the cross-section on two adjacent node points and multiplies it with the distance between these two node points δxi. This is a variational form of trapezoidal integration technique.

## Calculation of Constraints

### Setting up Linear Inequality constraints

Constraints c1R, c2R, c3(R) are set up as Linear Inequality constraints of the format fmincon requires it in - AR≤b where A is the matrix that contains the linear mapping between the design variables R and constraint values b.

### Setting up Nonlinear constraints

This is the most important aspect of this project as the computation of normal stress at each cross-section requires the use of numerical methods like Finite Element Methods to calculate it. The Area moment of Inertia for each cross-section along the length is,

| Iyyi= 14π(Routi4-Rini4) | Equation 3 |
| --- | --- |

Then, the nominal force which varies linearly along the length of the wing is calculated at each Node point as

| 0Lf dx= 2.5*Operational Load(N)2 ;Fnomi=f(1- xi/L ) N | Equation 3 |
| --- | --- |

The operational load considered in this problem is Operational Weight - 500kg ×9.81 m/s2

### Loading with Uncertainty

Loading in this project has uncertainty of the form given by the following Equation 4.

| fx,ξ=Fnomx+δf(x,ξ) |  |
| --- | --- |
| δfx,ξ= n=1n=4ξncos2n-1πx2L | Equation 4 |
| ξn~N0,Fnom010n |  |

This perturbed vector of forces are inputs to computing the normal stress at each nodal location and hence it is extremely important to capture the entire statistics of the constraint rather than just its behavior under nominal loading conditions. These perturbations could lead to both over loading and under loading conditions, and the optimized design should be able to handle this range of loading conditions that arise as a result of perturbations. The statistics of primary interest are the Estimated value of normal stress and its standard deviation. They can be computed using Equations 5,6, and 7.

| μsf,ξ1,ξ2,ξ3,ξ4= -∞∞-∞∞-∞∞-∞∞sf,ξ1,ξ2,ξ3,ξ4P1ξ1;f,σ1dξ1 P2(ξ2;f,σ2) dξ2 P3ξ3;f,σ3 dξ3 P4ξ4;f,σ4dξ4 |  |
| --- | --- |
| sf,ξ-stress f-nominal force ξ-perturbations Pξ;x,σ-probability model for perturbations σ-standard deviation of perturbations | Equation 5 |

| μs2f,ξ1,ξ2,ξ3,ξ4= -∞∞-∞∞-∞∞-∞∞sf,ξ1,ξ2,ξ3,ξ42P1ξ1;f,σ1 dξ1P2ξ2;f,σ2dξ2 P3ξ3;f,σ3dξ3 P4(ξ4;f,σ4)dξ4 |  |
| --- | --- |
|  | Equation 6 |

| σs= (μs2f,ξ1,ξ2,ξ3,ξ4-μ2(f,ξ1,ξ2,ξ3,ξ4)) | Equation 7 |
| --- | --- |
| σs-standard deviation of stress |  |

In this problem, the perturbations are modeled as Gaussian white noise given in Equation 4. On top of that, an oscillatory behavior is added to these perturbations using a cosine function along the length of the spar and it varies in amplitude scaled by these perturbations. These integrals are not trivial integrals, and it requires special integration techniques to carry them out. Perturbations to forces can be added and mean stress can be computed using Monte-Carlo sampling technique to keep the mathematics simple, but it drives the cost of the constraint up to a large extent. Hence stochastic collocation technique is preferred to perform the integration effectively, and as accurately as possible. The integrals reduce to this form for this 4-dimensional system:

| μsf= 1ππππk1=1mw(k1)k2=1mw(k2)k3=1mw(k3)k3=1mw(k4) s2σ1ξ1k1 cosπx2L+2 σ2ξ2k2cos3πx2L+2σ3ξ3k3cos5πx2L+2σ4ξ4k4cos7πx2L+fnom |  |
| --- | --- |
|  | Equation 8 |

| μs2f= 1ππππk1=1mw(k1)k2=1mw(k2)k3=1mw(k3)k3=1mw(k4) s22σ1ξ1k1 cosπx2L+2 σ2ξ2k2cos3πx2L+2σ3ξ3k3cos5πx2L+2σ4ξ4k4cos7πx2L+fnom |  |
| --- | --- |
|  | Equation 9 |

(ξnki,wki)≡(Quadrature points of random variable ξ, Quadrature integration weights)

The nonlinear constraint is now computed as c4R=μs+6σs≤σmax(yield stress). Incorporating these statistics into the constraint calculations results in a robust design which can handle uncertain loading conditions. These conditions are more likely to occur in practical scenarios.

# Setting up Optimization Problem

With the objective and the constraints set up, it is now important to calculate accurate gradients of the objective and the constraints for successful optimization of the design. Complex step gradient method is used to compute the gradients of the objective and the inequality constraints. With a very small complex step h=10-60 ,the round off errors caused by the central difference approximation can be avoided.

Two functions, Calc_objGradX and Calc_consJac(X,Nnodes,L,E,F,σmax) are used to compute the gradients of the objective function and the nonlinear constraint function.

Number of elements - 30

Initial point chosen - 0.050.0415     0.050.04150.05 ⋯⋯⋯0.050.041562×1'

Options chosen for fmincon –

- Choice of Algorithm – ‘Sequential Quadratic Programming (SQP)’

- Gradients – Provided and computed using Complex Step Algorithm

Initial weight of the spar was 29.320484235953565 kg

Initial Design choice is shown in Figure 2

Figure 2. Initial Design Parameterization

After optimization is performed, the optimal profile of the cross-section is shown in Figure 3

Figure 3. Optimal design of Spar Cross-section

Figure 4. First Order Optimality condition

Figure 4 shows the first order optimality condition to decrease and this conveys the fact that the optimizer has converged to a local minimizer which also satisfies the constraints.

The final weight of this optimal design is 8.837485790505804 kg. This is a 69.86% reduction of weight from the initial design.

# Results

A convergence analysis was performed to find if the mass estimate computed in this project improves as the number of elements (Nelem) are increased. As more cross-sections are added along the length elements, approximate volume calculated using the trapezoidal integral method improves as δxi decreases and a small increase in the mass computed can be noticed as shown in Figure 5.

Figure 5. Mass estimates (v) number of elements

Optimized profile undergoes the following mean displacement and mean normal stress displayed in Figure 6 and Figure 7 due to uncertain loading conditions.

| Figure 6. Displacement under uncertain loading | Figure 7. Stress under uncertain loading |
| --- | --- |

It is important to notice the fact that, if the optimization was performed without this uncertainty, the mass of the final spar was approximately 38% lesser than the mass of spar obtained when considering the uncertain loading conditions. The performance of the optimal design achieved using deterministic loading conditions were exactly like the plots of  μ+6σ{u,stress} shown in Figure 6 and Figure 7. Another important aspect to note is that the increase in mass has particularly happened near the root of the spar where there is maximum load. Since there is a possibility of maximum uncertainty at the root, the addition of more mass at the root makes intuitive sense as well.

This profile for the spar has not just decreased its mass but has also adhered to all the manufacturing constraints specified for the design of the spar. This is the final optimized profile achieved using this parameterization for the choice of design variables Routi, Rini i:11n. This is a robust design as opposed to the optimal design achieved without the consideration of uncertain loading conditions.

# Appendix

The MATLAB code written for this project has been attached in this section.